Command String: FAST=1WAKE=2*BOOT="!Lbt"!SAVEbt=<u0!X<r0!S>u0!E!O!R10

FUNCTION - sets up GoPro to automatically boot and record when USB power is applied, and safely shut down when power is removed.

Command string interpretation –

- FAST=1: Enables a faster GoPro Labs system boot sequence.

- WAKE=2: Configures the camera to automatically power on (wake up) when an external USB power source is connected.

- *BOOT="!Lbt": Tells the camera to load and run a persistent script named Lbt (Labs boot) from the SD card every time the camera boots up.

- !SAVEbt=...: Saves the following macro/script content directly to the Lbt file on the SD card so it runs automatically on future power-ons.

- <u0: Conditional check—if USB power is not present, exit the script (allowing normal manual camera use).

- !X: Cancel current operations. As soon as the condition above is met (power is cut), this action command tells the GoPro to immediately abort whatever it is doing.

- <r0: Resets or checks USB connection status/internal register.

- !S: Start a continuous loop for monitoring power and recording states.

- >u0: Conditional action based on USB power state.

- !E: End the current video capture/recording.

- !O: Power off the camera.

- !R10: Loop back (R) with a 10-second delay/condition check to repeat or manage the shutdown timing after power loss.