Command String: mVr5p50fLe0wAsMi1M8S90d10oC0oV1oW1oS1!SM2-2H10!R

FUNCTION – sets up GoPro in motion detection mode for camera trapping.  Detection is by comparison of consecutive video frames.

Command string interpretation –

📹 Video & Protune Settings

- mV: Sets the camera to Video mode.

- r5: Selects the 5.3K resolution.

- p50: Configures the frame rate to 50 fps.

- fL: Sets the lens FOV to Linear.

- e0: Turns electronic image stabilization (HyperSmooth) OFF.

- wAsM: Sets White Balance to Auto (wA) and Sharpness to Medium (sM).

- i1M8: Locks the ISO range between a Minimum ISO of 100 (i1) and a Maximum ISO of 800 (M8).

- S90: Sets the shutter control/angle (typically fixed or auto-locked rules depending on lighting).

- d10: Configures the colour depth to 10-bit color.

⚙️ Camera Option Settings

- oC0: Sets Camera Auto Off to Never (keeps the camera from falling asleep on its own).

- oV1: Sets the system beep volume to 10%.

- oW1: Keeps Wireless/Wi-Fi connection ON.

- oS1: Sets the LCD screen auto-off timeout to 1 minute (saves power while monitoring).

🏃‍♂️ Action Scripting (The Motion Trigger)

The second half of the code utilizes specialized action and conditional triggers:

- !SM2-2H10: This translates to Start on Motion Detection (!S). It sets a motion sensitivity profile (M2-2) and specifies Hold for 10 seconds (H10), meaning the GoPro will record for at least 10 seconds after motion is detected before checking if the motion has stopped.

- !R: Repeat/Restart the command. This loops the motion detection action indefinitely, ensuring that every time new motion occurs, a new video clip is generated.